X 8. A

X 4. B

X 4. C

X 8. D

X 24. E

X 12. F

FITTINGS

# Intuition 1500 open bookcase unit assembly instructions. IBU1516

STEP 1.

- Lay 1 end panel on its side as per diagram right.

- Insert 2 x metal dowels (F) into end panel for the middle shelves.

- Insert 24 wooden dowels (E) into end, top and bottom.

- Insert 2 long metal dowels (D) into top edge of all 4 vertical panels. Top edge determined by veneer crowns.

- Drop shelves onto dowels, middle shelf has metal cams. Make sure veneered edge of shelf faces front of unit.

- Tighten cams on middle shelf.

STEP 4.

- Align bottom panel with bottom of unit.

- Make sure back panels fit into groove in bottom.

- Fix bottom to vertical panels using 8 x screws  (A) provided.

- Align KD fittings in top with metal dowels (D) in vertical panels.

- Push top to fit flush. Make sure back panels fit into groove in top.

- Using a pozi screw driver tighten each KD fitting in top.

- Push plastic feet (B) into holes in bottom and cover caps (C) on inside of bottom.

- Rotate cabinet into correct position.

STEP 2.

- Drop back panel into groove in end panel.

- Insert wooden and metal dowels (D, E&F) into divider panel (both sides and end) as step 1.

- Drop divider panel onto shelves aligning dowels with holes in shelves and back panel into groove.

- Tighten cams in middle shelf.

STEP 3.

- Continue building cabinet as described in steps 1 and 2, use remaining divider panel then end panel to finish.

- Tighten cams on each middle shelf as you go.

17/4/2002. SHEET 7

4 x cams