X 4

X 4

X 4

# Intuition Floating returns assembly instructions

### IKRT 1106. ICRT1107. IMPT0506. IRT/FP

STEP 2.

Align metal spacers with holes in tops and fix in position using the mushroom headed screws provided.

STEP 1.

- Undo small runner screws on underside edges of drawers (retain screws).

- Lift drawers/shelf up and pull to remove.

- Drill out part bored holes in underside of top (Use
13mm diameter drill.

STEP 4.

Fix tops to cabinets using the M12 x 35 button socket cap screws and large washers provided.

1/5/2004  SHEET 4

#### X 16

FITTINGS

#### X 2

STEP 3.

Fix linking plates to underside of return top (crescent or kidney) using the mushroom headed screws provided.

STEP 5.

Replace all drawers/shelf

following step 1 in reverse.

STEP 6.

Fix linking plates on return top (crescent or kidney) to underside of desk top using the mushroom headed screws provided.